Chris Conlon

Network Design Proposal

For Continental Noodle Organization

Maverick Network Consulting Inc.

Requirements:

| Node | Name | V-Cord | H-Cord |
| --- | --- | --- | --- |
| N1 | New York | 4997 | 1406 |
| N2 | Los Angeles | 9213 | 7878 |
| N3 | Chicago | 5986 | 3426 |
| N4 | Dallas | 8436 | 4034 |
| N5 | Philadelphia | 5239 | 1506 |
| N6 | Washington, DC | 5622 | 1583 |
| N7 | Seattle | 6336 | 8896 |
| N8 | Miami | 8351 | 527 |
| N9 | Atlanta | 7260 | 2083 |
| N10 | Boston | 4422 | 1249 |
| N11 | Denver | 7501 | 5899 |
| N12 | Rochester, NY | 4913 | 2195 |
| N13 | Boulder, CO | 7460 | 5957 |
| N14 | Vancouver, BC | 5980 | 9032 |
| N15 | Toronto | 4981 | 2488 |
| N16 | Mexico City, Mexico | 11293 | 3419 |
| N17 | Rio de Janeiro | 17663 | -9665 |

Several sites V&H coordinates were derived from Lat/Lon Data:

City				  Latitude	 Longitude         V          H

Boulder                   	  40° 00' N     105° 16' W     7460     5957

Mexico City              	  19° 24' N     099° 12' W     11293   3419

Buenos Aires              	  34° 35' S      058° 29' W     17663   -9665

Toronto, Canada		  43° 39’ N	  079° 23’ W     4981    2488

Vancouver, BC, Canada	  49° 16’ N	  123° 07’ W     5980    9032

- Each city has 11 branch locations which must be networked to a 12th backbone connected site.

- The backbone sites need to be connected via inter-city links which are arranged in a fully meshed backbone.

- Each city’s access network follows the same traffic pattern and quality, reliability, service, innovation, price and flexibility must be weighed evenly.

- Two designs must be submitted one being the least cost and the other being the best overall choice.

Local Access Network Designs (Intra-City)

Traffic Model for Each City (Using 24-48 traffic Generator Profile & restricting the algorithms to only use D56 links)

%TABLE SITES

%NAME++ TYPE+IDD+ VCORD+HCORD++PARENT++ POP LEVEL+TRAFOUT+ TRAFIN+

N1 E      1        5003        3782                  N1    1         1        2400     2400

N2 E      1        7557        3849                  N2    1         1        2400     2400

N3 E      1        6107        4833                  N3    1         1        4800     4800

N4 E      1        7360        2550                  N4    1         1        4800     4800

N5 E      1        7246        3623                  N5    1         1        2400     2400

N6 E      1        5047        2289                  N6    1         1        2400     2400

N7 E      1        5465        2524                  N7    1         1        4800     4800

N8 E      1        6409        2376                  N8    1         1        2400     2400

N9 E      1        5028        3194                  N9    1         1        4800     4800

N10 E     1        6806        3902                 N10   1         1        4800     4800

N11 E     1        5525        4096                 N11   1         1        2400     2400

N12 N     1        6113        2180                 N12   1         2      38400     38400

%TABLE LINETYPES

% SVTY+ TYPE   SPEED+++  FIXED_COST   DIST_COST1   DIST_COST2   DIST1

D56    LS      56000      500        5              3          200

Designs Generated:

|  | Cost | Reliability | Max Utilization | Avg. Hop Count |
| --- | --- | --- | --- | --- |
| CMST | $24,620.00 | 0.8943 | 0.386 | 2.8182 |
| Tour | $28,970.00 | 0.9926 | 0.386 | 3.2727 |

Although the Tour design has a 9.3 percent higher reliability it costs 17.6 percent more than the CMST design.  Both designs have less than a 50 percent maximum link utilization, which allows some flexibility for bandwidth increases as well as minimizing delay.  Additionally, the CMST design has a lower average hop count which reduces network delay.  All costs are based on 56K Leases Lines.

Figure 1 (CMST)

Figure 2 (CMST)

Figure 3 (Tour)

Figure 4 (Tour)

Conclusion Intra-City Design

For the local intra-city networks I recommend the CMST design.  It is both the cheapest and the least delayed network.  The Tour network is more reliable but not worth the significant jump in price.

(17 cities x $24,620.00) + (11 sites per city x $2000 per router) = $440,540

Backbone Network Designs (Inter-City)

- Fully meshed network between 17 cities

Leased Line Considerations

136 LLs @ 64Kbps =   $27,200

248,264 miles = $248,264

136 LLs @ 128Kbps =   $34,400

248,264 miles = $372,396

Frame Relay Design Consideration

Port Charges

13 cities x $1,925 1024Kbps Port + 4 int’l cities x $7,700 + 17 x $825 = $69,850 + $17,000 installation charge

CIR Charges

56/64kbps -> $112 x 136 PVCs = $15,232

Distance matrix between Cities

n1n2n3n4n5n6n7n8n9n10n11n12n13n14n15n16n17

n102443712136983206240710977471891627251818243234320915320

n20173912412375229296524971934258782822548231086216815566158

n3079965259517341184586848918517927177343516795548

n40128911811675111072115456601257682176211979255224

n501242363103266527115632421574239232320085284

n60232492654239414892971501235935118855211

n7027232175249310172167996121207223376877

n8060112641720120917412793123313054366

n9093612107441227223573213444963

n10017643381772251143022795427

n1101429231102134214335878

n120143721899620555507

n1301080134914545901

n140209424456972

n15020185555

n1604602

n170